Region 9 Army MARS Training Topics for “phone bridge” use
TRAINING TOPIC   –  Sending Messages by Voice
LESSON INTENT –  Part 2 - Introduction to the procedure of sending by voice
REFERENCES – ACP-125 Radio Telephone Procedures 



        AM-6 Annex J
EXPECTED TIME TO DELIVER – 55 Min.
1. Although most messages today are as text using a digital procedure, message by voice 
remain a core competency of MARS stations.  The example illustrates procedure for 
sending a message by voice.

Procedure for Sending a message by voice
Parts


Components
Line

Elements
The Heading
Call:




Messages Follows







FL 2&3
AAR9TC









THIS IS AAR9CB NUMBER 001









Transmission Instructions:
“Here are some specific instructions













   about transmitting this message”










RELAY TO AAA9RD




Preamble:



“Here is some information about this message, specifically, 









   how important it is, the date and time it was drafted which 









   tells you by when it should be delivered”









FL 5

ROUTENE TIME ZERO FOUR ONE NINER FOUR 









THREE ZULU MAY TWO ZERO ONE NINE




Address:



“This message is from this person or organization and is 









   intended for these other people or organizations”




Prefix:

FL 10

“This tells you how many groups, or words, are in the message. 








  (Not normally utilized except for coded message)










GROUPS TWELEVE
The Separation



FL 11

BREAK
The Text




FL 12

Subject Matter
“This is the body of the message”
The Separation



FL 13

BREAK
The Ending




FL 16

OVER
WXOBS VOICE TEMPLATE CROSS REFERENCE 
This message is transmitted by voice when means for text transmission are not available or when a transmission by voice is required to provide a timely dissemination of information. Upon receiving a voice message of this type, MARS stations must transcribe the voice message into a USMTF Text message using the cross reference provided below.
Index
          Type of Information
  Voice Example             USMTF Text Cross Reference
(Verbalized)   (Not  Verbalized)
	
	
	
	
	

	                                Security Warning 
	 UNCLASSIFIED 
	UNCLAS

	                                Exercise Warning 
	 EXERCISE 
	EXER/MARS COMEX//

	                                USMTF Type 
	 WEATHER OBSERVATION 
	MSGID/WXOBS/-//

	LOCATION 
	KILO ECHO WHISKEY ROMEO 
	WEATHLOC/ICAO:KEWR// 

	OBSERVATION TIME 
	TWO NINER ZERO EIGHT TWO NINER ZULU 
	OBSTIME/290829Z// 

	WIND DIRECTION 
	ONE ONE ZERO 
	WIND/110/10// 

	WIND SPEED 
	ONE ZERO 

	VISIBILITY 
	THREE STATUTE MILES 
	VSBY/3SM

	SKY COVER 
	SKY CLEAR 
	CLDLYR/-/LYR:SKC// 

	MAXIMUM TEMPERATURE 
	TWENTY 
	TEMP/20/20// 

	MINIMUM TEMPERATURE 
	TWENTY 

	ALTIMETER 
	TWO NINER DECIMAL NINER TWO INCHES MERCURY 
	ALTSTG/HG:29.92// 


AAR9TC - THIS IS - AAR9CB
UNCLASSIFIED

EXERCISE M9T

WEATHER OBSERVATION

LOCATION



KILO ECHO WHISKEY ROMEO

OBSERVATION 


TIME TWO NINER ZERO EIGHT TWO NINER ZULU

WIND DIRECTIONONE 

ONE ZERO

WIND SPEEDONE 


ZERO

VISIBILITY



THREE KILOMETERS

SKY COVERSKY 


CLEAR

MAXIMUM TEMPERATURE
TWENTY

MINIMUM TEMPERATURE
TWENTY

ALTIMETER



TWO NINER DECIMAL NINER TWO INCHES MERCURY
THE WAY AHEAD – In part 3 we will talk about the proper verbal procedures in receiving a voice message starting with the header and adhering to Speech Techniques and Rules we talked about in part 1.
US Army MARS Region 9
Training Topic – 23 APRIL 2019– Page 4
Prepared by AAA9R7

